PROJECT REPORT

ESP32 Industrial IoT

Monitoring & Control System

IoT · Industrial Automation · Embedded Systems · Wireless Monitoring & Control

| Project type | Student-built prototype |
| --- | --- |
| Author / Team | To be updated |
| Institution | To be updated |
| Guide | To be updated |
| Version / Date | To be updated |

Table of Contents

1. Introduction	3

2. Problem Statement	3

3. Objectives	3

4. System Overview	4

5. Hardware Components	4

6. Software Architecture	5

7. ESP-NOW Communication	7

8. Web Dashboard	8

9. Hardware Connections	10

10. System Working	10

11. Testing and Validation	11

12. Results	12

13. Limitations	12

14. Future Scope	13

15. Conclusion	13

16. References	13

Status labels used in this report. Implemented: present in the current design, as described in the project information (firmware source is not yet included in the repository). Tested: validated on hardware and recorded in Section 11; none recorded yet. Planned: near-term work not yet done (validation, documentation of Sender pins, photographs). Future: possible extensions, not implemented. Items that depend on the firmware source are marked To be verified against source.

# 1. Introduction

This project is an ESP32-based system for monitoring environmental and object-detection parameters, with local relay control. It uses two ESP32 boards:

- Sender ESP32 reads a DHT11 (temperature, humidity), an MQ-2 gas sensor and two IR sensors.

- Receiver ESP32 receives the data over ESP-NOW, shows it on a 1.3” OLED, hosts a web dashboard on its own Wi-Fi access point, and drives two relays.

The design follows the basic Industrial IoT pattern: sensors collect field data, wireless communication transfers it, and a local digital interface provides monitoring and control.

# 2. Problem Statement

Monitoring temperature, humidity, gas level and object presence usually needs wired sensor runs or network infrastructure such as a router or cloud service. This project builds a low-cost alternative from common hardware: a wireless sensor node feeding a local display, a web dashboard and switching outputs, with no router or internet connection needed for local monitoring.

# 3. Objectives

- Wireless sensor data transmission (ESP-NOW)

- Real-time temperature and humidity monitoring

- Gas-level monitoring (raw sensor level, not calibrated concentration)

- IR-based object detection/status monitoring

- Local OLED display

- Web-based monitoring from a laptop or smartphone

- Live graphs of gas and temperature trends

- Relay control from the dashboard

- Two-ESP32 distributed architecture

# 4. System Overview

Figure 1 – Overall system architecture.

The Sender collects sensor data and transmits it over ESP-NOW. The Receiver passes it to three outputs: the OLED, the web server (reached through the ESP32 Wi-Fi access point) and the relay control.

Table 1 – Implementation status

| Capability | Implemented | Tested | Planned / Future |
| --- | --- | --- | --- |
| Sensor acquisition (DHT11, MQ-2, IR 1, IR 2) | Yes | Not yet | Validation planned |
| ESP-NOW transmission and reception | Yes | Not yet | Validation planned |
| OLED display | Yes | Not yet | Validation planned |
| Wi-Fi AP and web dashboard with live charts | Yes | Not yet | Validation planned |
| Relay control from dashboard | Yes | Not yet | Validation planned |
| Sender GPIO documentation, photographs, screenshots | – | – | Planned |
| Calibrated gas measurement, cloud, authentication, alerts | No | – | Future (Section 14) |

# 5. Hardware Components

Table 2 – Hardware components

| # | Component | Qty | Role |
| --- | --- | --- | --- |
| 1 | ESP32 development board | 2 | Sender and Receiver |
| 2 | DHT11 temperature and humidity sensor | 1 | Temperature, humidity |
| 3 | MQ-2 gas sensor | 1 | Raw gas sensor level |
| 4 | IR sensor | 2 | Object detection/status |
| 5 | 1.3” OLED display (I2C, U8g2) | 1 | Local display on the Receiver |
| 6 | Relay module | 1 | Relay 1 and Relay 2 outputs |
| 7 | Jumper wires, breadboard / prototype wiring | – | Interconnects |
| 8 | USB power / programming connections | – | Power and flashing |

Relay module channel count, OLED controller variant and I2C address, sensor supply voltages and load ratings: Not specified in the current implementation.

Table 3 – Receiver pin configuration (confirmed)

| Function | GPIO |
| --- | --- |
| OLED I2C SDA | 21 |
| OLED I2C SCL | 22 |
| Relay 1 | 25 |
| Relay 2 | 26 |

Sender pins (DHT11, MQ-2, IR Sensor 1, IR Sensor 2): To be updated according to final hardware wiring.

Photographs (placeholders). Hardware setup, Sender ESP32, Receiver ESP32: [Insert actual hardware photographs here]

# 6. Software Architecture

Table 4 – Technology stack

| Layer | Technology |
| --- | --- |
| Hardware | ESP32, DHT11, MQ-2, IR sensors, OLED, relay module |
| Communication | ESP-NOW, Wi-Fi Access Point |
| Embedded software | Arduino framework, C/C++ |
| Libraries | WiFi.h, esp_now.h, WebServer.h, Wire.h, U8g2lib.h, esp_wifi.h |
| Web | HTML, CSS, JavaScript, Chart.js (live temperature and gas trend graphs) |

## 6.1 Sender ESP32

Figure 2 – Sender ESP32 block diagram.

## 6.2 Receiver ESP32

Figure 3 – Receiver ESP32 block diagram.

## 6.3 Firmware flow

Figure 6 – Software flowchart.

The Sender initializes, then repeatedly reads sensors, builds a packet and transmits it. On the Receiver, initialization is followed by independent runtime functions rather than one serial sequence:

- The ESP-NOW receive callback updates the stored sensor values.

- The OLED update shows the latest stored values.

- The web server serves two separate request handlers: GET /data returns the stored values as JSON, and the relay command handler sets GPIO 25 / GPIO 26. Relay switching does not depend on the /data response.

The firmware is structured so that the web server keeps responding while sensor updates arrive.

## 6.4 Implementation details

Table 5 – Firmware details

| Item | Current documentation | Status |
| --- | --- | --- |
| Receiver GPIOs | OLED SDA 21, SCL 22; Relay 1 = 25, Relay 2 = 26 | Confirmed |
| Sender GPIOs | – | To be updated according to final hardware wiring |
| Data structure | temp, hum, gas, ir1, ir2 | Confirmed |
| JSON structure | temp, hum, gas, ir1, ir2, r1, r2 (approximate) | Field types to be verified |
| Data endpoint | /data | Confirmed |
| Relay endpoints | – | To be verified |
| Receiver Wi-Fi | Access point, SSID ESP32_DASH | Exact mode setting to be verified |
| Wi-Fi channel, Sender and Receiver MAC | – | To be verified |
| ESP-NOW receive callback | Updates stored sensor values | Details to be verified |
| OLED initialization | U8g2, I2C on GPIO 21/22 | Constructor / address to be verified |
| Dashboard refresh interval | Periodic automatic refresh | Interval to be verified |
| Cache handling | Cache prevention used | Method to be verified |
| Chart.js | Gas and temperature trend charts | Point count, CDN vs. local loading to be verified |
| Relay logic | Active-low for this relay module | Confirmed |

# 7. ESP-NOW Communication

ESP-NOW is an Espressif protocol that lets ESP32 devices exchange short packets directly, without a Wi-Fi router. It has low protocol overhead and suits local sensor-to-controller links. Range and reliability have not been measured for this project.

Figure 4 – ESP-NOW communication flow.

Table 6 – Data packet fields

| Field | Type | Meaning |
| --- | --- | --- |
| temp | float | Temperature from the DHT11 |
| hum | float | Relative humidity from the DHT11 |
| gas | int | Raw MQ-2 reading (Gas Sensor Level) |
| ir1 | int | IR Sensor 1 value |
| ir2 | int | IR Sensor 2 value |

Sender and Receiver must use the same Wi-Fi channel for ESP-NOW to work. Channel selection, peer MAC handling and the meaning of each IR value: to be verified against source.

# 8. Web Dashboard

The Receiver hosts a local web server (WebServer.h) on its own access point, so the data path works without internet access. Several devices can connect to the access point, subject to the practical limits of the ESP32; no maximum user count is claimed. If Chart.js is loaded from a CDN, the client needs internet access for the charts; to be verified.

| Access point | Value |
| --- | --- |
| SSID | ESP32_DASH |
| Password | 12345678 (defined in firmware; change before deployment) |

## 8.1 Layout

Table 7 – Dashboard layout

| Zone | Content |
| --- | --- |
| Top | Project title / system status |
| Metrics | Temperature, Humidity, Gas Sensor Level, IR Sensor 1, IR Sensor 2 |
| Graphs | Gas Level Trend, Temperature Trend |
| Controls | Relay 1, Relay 2 |

The layout is responsive for laptop and mobile screens, with automatic refresh.

Screenshots (placeholders). Dashboard on laptop and on smartphone: [Insert actual dashboard screenshots here]

## 8.2 Data flow and API

Figure 7 – Web dashboard data flow.

The browser sends GET /data; the Receiver returns the current values as JSON; JavaScript updates the cards and adds temperature and gas values to the charts. Cache prevention makes the browser fetch current data instead of a cached response.

Table 8 – /data response fields

| Field | Description |
| --- | --- |
| temp | Temperature (DHT11) |
| hum | Humidity (DHT11) |
| gas | Raw MQ-2 Gas Sensor Level |
| ir1 | IR Sensor 1 value |
| ir2 | IR Sensor 2 value |
| r1 | Relay 1 state |
| r2 | Relay 2 state |

Engineering note. The MQ-2 output shown in this project represents the sensor reading/level used for monitoring. Accurate gas concentration in PPM requires proper sensor calibration, load resistance considerations, environmental compensation, and gas-specific calibration.

## 8.3 Relay control

Figure 8 – Relay control flow.

Relay commands from the dashboard are handled by the Receiver web server separately from the /data request, and set GPIO 25 (Relay 1) and GPIO 26 (Relay 2). In the relay module used in this project the outputs are active-low: LOW = relay ON, HIGH = relay OFF. This applies to this project’s module only. Relay endpoint names: to be verified.

## 8.4 OLED display

Figure 9 – OLED display data flow.

The OLED shows receiver status, temperature, humidity, gas value, IR1 and IR2, using U8g2 over I2C (SDA GPIO 21, SCL GPIO 22).

OLED photograph (placeholder). [Insert actual OLED photograph here]

## 8.5 Network notes

The access point is local; dashboard traffic stays within the ESP32 network. This is not production-grade security. Change the password before serious use and avoid hard-coding credentials in a public repository.

# 9. Hardware Connections

Figure 5 – Hardware connection diagram.

Sender pin assignments: final GPIO mapping to be added from actual hardware wiring. Power arrangement beyond USB and relay load wiring: Not specified in the current implementation.

# 10. System Working

- The Sender ESP32 powers up.

- Sensors are initialized.

- The Sender reads temperature and humidity from the DHT11.

- The Sender reads the MQ-2 gas sensor.

- The Sender reads IR Sensor 1 and IR Sensor 2.

- Readings are stored in a structured data packet.

- The packet is transmitted using ESP-NOW.

- The Receiver ESP32 receives the packet.

- The Receiver updates its sensor variables.

- The latest values are shown on the OLED.

- The Receiver web server provides sensor data at /data.

- A laptop or phone browser requests the latest data.

- JavaScript updates the metric cards.

- Temperature and gas values are added to the live charts.

- The user controls Relay 1 and Relay 2 from the dashboard; the relay command is handled separately from data requests.

Figure 10 – Complete end-to-end system flow.

The Sender reads periodically, the Receiver updates when packets arrive, the dashboard polls /data, and the OLED shows the latest received values. Intervals: to be verified against source.

# 11. Testing and Validation

No tests have been performed or recorded. Every entry below is Pending Validation; actual results and PASS/FAIL status will be entered only after hardware testing.

Table 9 – Test matrix

| ID | Test Description | Expected Result | Actual Result | Status | Remarks |
| --- | --- | --- | --- | --- | --- |
| T01 | Sender ESP32 startup | Boots; sensors and ESP-NOW initialize | To be filled | Pending Validation |  |
| T02 | DHT11 temperature reading | Plausible temperature reported | To be filled | Pending Validation |  |
| T03 | DHT11 humidity reading | Plausible humidity reported | To be filled | Pending Validation |  |
| T04 | MQ-2 sensor reading | Gas Sensor Level changes with gas exposure | To be filled | Pending Validation | Warm-up needed; uncalibrated |
| T05 | IR Sensor 1 | Value changes on object presence | To be filled | Pending Validation |  |
| T06 | IR Sensor 2 | Value changes on object presence | To be filled | Pending Validation |  |
| T07 | ESP-NOW communication | Receiver gets packets from Sender | To be filled | Pending Validation |  |
| T08 | Receiver OLED display | OLED shows latest values | To be filled | Pending Validation |  |
| T09 | Web dashboard access | Dashboard loads via ESP32_DASH | To be filled | Pending Validation |  |
| T10 | Live dashboard update | Cards refresh automatically | To be filled | Pending Validation |  |
| T11 | Gas graph update | Gas trend chart updates | To be filled | Pending Validation |  |
| T12 | Temperature graph update | Temperature trend chart updates | To be filled | Pending Validation |  |
| T13 | Relay 1 control | Relay 1 toggles from dashboard | To be filled | Pending Validation | Active-low |
| T14 | Relay 2 control | Relay 2 toggles from dashboard | To be filled | Pending Validation | Active-low |
| T15 | Mobile dashboard access | Dashboard usable on smartphone | To be filled | Pending Validation |  |

Table 10 – Troubleshooting

| Problem | Possible causes |
| --- | --- |
| ESP-NOW data not received | Different Wi-Fi channel; incorrect MAC address; ESP-NOW initialization failure; sender/receiver not configured correctly |
| Dashboard shows -- | Browser cannot reach /data; Receiver not receiving ESP-NOW data; web server issue; browser cache; network connection issue |
| OLED not displaying | Incorrect I2C wiring; wrong OLED address; incorrect SDA/SCL; library configuration |
| Relay not switching | Incorrect GPIO; active-low logic misunderstanding; relay power issue; wiring problem |
| Gas value unstable | MQ-2 warm-up; environmental changes; sensor characteristics; lack of calibration |

# 12. Results

No numerical results are reported. The prototype is designed to demonstrate the capabilities below; evidence will be added after validation.

| Capability | Evidence |
| --- | --- |
| Sensor data acquisition | [Add Serial Monitor screenshot] |
| Wireless transmission over ESP-NOW | [Add Serial Monitor screenshot] |
| Local visualization on the OLED | [Add OLED photograph / capture] |
| Web dashboard with live charts | [Add dashboard screenshots] |
| Relay control | [Add photograph or video] |
| Laptop and mobile access | [Add screenshots] |

# 13. Limitations

- MQ-2 values are raw sensor levels, not calibrated PPM measurements, unless calibration is performed.

- ESP-NOW range depends on environment, antenna, interference and hardware; it has not been measured.

- This is a prototype and is not certified for industrial safety applications.

- Relay outputs must not be connected to dangerous loads without appropriate electrical protection and isolation.

- The Wi-Fi password is defined in firmware and should be changed for deployment.

- Sensor accuracy depends on sensor quality, calibration, environment and wiring.

- No database or cloud storage is used.

- The project focuses on local monitoring and control.

# 14. Future Scope

None of the following is implemented.

| Area | Possible improvements |
| --- | --- |
| Connectivity | MQTT, cloud IoT dashboard, remote monitoring, multiple sensor nodes |
| Data | Database logging, historical storage, data export, SD card logging |
| Security | User authentication, HTTPS, better network security |
| Sensing | Sensor calibration, industrial-grade sensors, additional sensors |
| Automation | Alarm/buzzer, automatic relay rules, emergency shutdown system |
| Alerts and apps | Email/Telegram alerts, mobile application |
| Reliability | OTA firmware updates, watchdog and fault recovery |
| Hardware | Enclosure, PCB, industrial power supply |

# 15. Conclusion

The project demonstrates a local ESP32-based IoT architecture combining sensor acquisition, ESP-NOW wireless communication, local processing, OLED visualization, web-based monitoring and relay control. It is a prototype: it makes no claim of industrial certification, calibrated gas measurement or production readiness. It provides a base that can be extended toward larger IoT and automation applications through the items in Section 14.

# 16. References

- Espressif Systems, ESP32 Series Datasheet and Technical Reference Manual. https://www.espressif.com/en/support/documents/technical-documents

- Espressif Systems, ESP-NOW, ESP-IDF Programming Guide. https://docs.espressif.com/projects/esp-idf/en/latest/esp32/api-reference/network/esp_now.html

- Espressif Systems, Arduino core for the ESP32 documentation. https://docs.espressif.com/projects/arduino-esp32/

- olikraus, U8g2 library documentation. https://github.com/olikraus/u8g2/wiki

- Aosong, DHT11 datasheet (version to be updated).

- Hanwei Electronics, MQ-2 gas sensor datasheet (version to be updated).

- Chart.js Contributors, Chart.js documentation. https://www.chartjs.org/docs/